Product Requirements Document

DASHBOARD - Display

| Issued by | Role/Department | Date |
| --- | --- | --- |
| Alessandro Zingaretti | Electronics Team Member | 29/09/2025 |

REVISION TABLE

| Version | Date | Issued by | Description |
| --- | --- | --- | --- |
| A-1 |  |  |  |
| A-2 |  |  |  |
| A-3 |  |  |  |
| B-1 |  |  |  |
| B-2 |  |  |  |
| B-3 |  |  |  |
| B-4 |  |  |  |

TABLE OF CONTENTS

GENERAL PRODUCT DESCRIPTION	2

INPUT AND OUTPUT	3

ENVIRONMENT	4

INTERFACE	4

DESIGN	4

SOFTWARE ARCHITECTURE AND DATA PROCESSING	5

ELECTRICAL HARDWARE AND SENSORS	6

PRODUCT VALIDATION / QUALIFICATION	7

# GENERAL PRODUCT DESCRIPTION

| Product Description |  |
| --- | --- |
| Key Features/ Functionality | Showing car parameters on the steering wheel display (car speed, critical temperatures, R2D, SOC ...) |
| HV/LV | LV |
| Software | Nextion Editor, STM32CubeIDE |
| Software Language | C/C++, Nextion language |
| Other components Compatibility, Ecosystem, etc. | 74HC165 (shift register), MCP2551 (CAN-Bus transceveir) |
| Installation (where) | NX4832T035, STM32F303K8T6 |
| Dimensions and Tolerances | Nextion: 100.5 (L)×54.94 (W)×5.45 (H) (.X±,20) Stm32: |
| Software Connections | CANBus, UART |
| Weight | Nextion: 38.2g |

# INPUT AND OUTPUT

| Product Description |  |
| --- | --- |
| Data Input (CAN/Bluetooth….) | Stm: CAN-BUS Nextion: Serial |
| Signal Input | Stm: GPIO (Shift register, buttons) |
| Data Output (CAN/Bluetooth….) | Stm: CAN-bus, Serial |
| Signal Output |  |

# ENVIRONMENT

| Operating Environment |  |
| --- | --- |
| Where in the car | Steering Wheel |
| Temperature Range | Nextion: (-20,+70)°C Stm: (-40,+85)°C |
| EMI (vulnerability to electromagnetic interference) |  |

# INTERFACE

| User Interface |  |
| --- | --- |
| Communication protocol | USART (115200 baud rate), CANbus (500kbits) |
| Communication Input |  |
| Communication output |  |
| Connector |  |
| Visual Interface (Screen size and type, LEDs) | Nextion: 100.5 (L)×54.94 (W)×5.45 (H) (.X±,20) |
| External Software interface | STM32CubeIDE |

# DESIGN

| Renderings |  |
| --- | --- |
| Color, Material, and Finish (CMF) |  |
| Logo size and placement |  |

# SOFTWARE ARCHITECTURE AND DATA PROCESSING

Block Diagram of Data Flow

The code is used to manage the communication between the car and the user interface on the steering wheel. (display and buttons) The software receive from CAN-bus the data to be displayed on the display. The data to be displayed are: SoC, inverter and motor temperature, speed,…. Once received and unpacked, they must be sent via serial to the display. In addition, the map selected by the hand controller is displayed. The value of the hand controller is managed by a shift register (see above for the datasheet) and has seven possible positions. Each position corresponds to a map for engine control, which the driver can choose by turning the knob. The code reads the value of the selected map and send it both via CAN (0x040) and via serial for the display. In particular: Hand controller’s Position; Display message; CAN message. 1; Map 1 (0%); 0. 2; Map 2 (11-30%); 1. 3; Map 3 (30%-60%); 2. 4; Map 4 (60%-100%); 3. 5; Error: Map not available!; 7. 6; Error: Map not available!; 7. 7; Error: Map not available!; 7.

Button 1 This button manages the R2D (EV regulation 4.11). When the car reads the button pressed and the brake message (0x012) high, the R2D parameter (0x016) is set high. Brakes (0x012); Button 1; R2D (0x016). 0; 0; 0. 0; 1; 0. 1; 0; 0. 1; 1; 1. Button 2 Pressing this button there will be a totaly soft reset.

| Parameters Definition |  |
| --- | --- |
| FAN speed map |  |

# PRODUCT VALIDATION / QUALIFICATION

| Data Test |  |
| --- | --- |
| Visualization Test |  |
| Ergonomics Test |  |
| Utility Test |  |